### Overview

The code manages communication between Rocrail (a model railroad control software) and the Märklin CAN bus via a two-core ESP32, providing translation between TCP and CAN. It consists of 2 main tasks and 1 optional debug task.

This code has been forked from Christophe Bobille’s ESP32_rocrail_can_tcp_gateway. Please see the full presentation of the project at https://www.locoduino.org/spip.php?article361

Changes/improvements include a Wi-Fi manager with OTA capability and debugging via telnet. The manager will set up an access point to input Wi-Fi credentials on first start up. Performance improvements come from streamlining the code and reducing the number to tasks from 4 to 2. (CAN and Wi-Fi are both half duplex, so separate send/receive tasks are unnecessary). ChatGPT and Claude were used extensively for coding assistance.

### Key Components

- CAN Communication:

  - The ESP32 communicates with a Marklin CAN bus using the ACAN_ESP32 library.

  - The CANHandlerTask receives and sends CAN messages.

- TCP Communication:

  - The TCPHandlerTask handles the reception of messages from Rocrail over TCP and sends the received CAN messages back to Rocrail via TCP.

- WiFi Setup:

  - The WiFiManager library is used to handle WiFi connections.

  - A WiFiClient is used to manage the TCP connection with Rocrail.

- Queues:

  - Two main queues handle the transfer of data between tasks:

    - canToTcpQueue: CAN messages to be sent over TCP.

    - tcpToCanQueue: TCP data to be converted and sent over CAN.

  - An optional debug queue (debugQueue) is used for verbose logging.

### Task Setup

- CANHandlerTask:

  - Reads CAN messages from the CAN bus and places them in the canToTcpQueue.

  - Uses ACAN_ESP32::can.receive() to receive frames.

  - Has a small delay to avoid busy-waiting.

  - Receives data from the TCP client, reading exactly 13 bytes (as per Rocrail's expected frame size).

  - Sends the received data to the tcpToCanQueue for conversion to CAN frames.

  - A single task is sufficient to handle sends and receives because CAN is a half-duplex protocol and cannot send and receive simultaneously.

    - Reading from CAN is prioritized.

- TCPHandlerTask:

  - Takes CAN frames from canToTcpQueue and sends them over the TCP connection.

  - It checks if the TCP client is connected, and if not, it attempts to restart after a delay.

  - Uses client.write() to send the data.

  - Receives data from the TCP client, reading exactly 13 bytes (as per Rocrail's expected frame size).

  - Sends the received data to the tcpToCanQueue for conversion to CAN frames.

  - A single task is sufficient to handle sends and receives because Wi-Fi on the ESP32 is half-duplex and cannot send and receive simultaneously.

    - Reading from TCP is prioritized.

- debugFrameTask (optional):

  - This task is for verbose output, sending CAN frame data to a debug queue and logging it using debugFrame().

  - Only included if VERBOSE is defined.

### Task Synchronization

- Queues are used for communication between tasks. Each task works asynchronously, pushing and pulling data between the CAN and TCP layers.

- Core Pinning:

  - CAN task (CANHandlerTask) is pinned to Core 0 (APP core).

  - TCP task (TCPHandlerTask) is pinned to Core 1 (COMM core).

- Priority:

  - Higher priority (5) for tasks that handle data (TCP and CAN).

    - Reading of data has higher priority within their respective tasks

  - The optional debug task has the lowest priority (1).

### Flow of Data

- CAN to TCP:

  - CANHandlerTask listens to the CAN bus and sends incoming messages to canToTcpQueue.

  - TCPHandlerTask receives messages from the queue and sends them over TCP to Rocrail.

- TCP to CAN:

  - TCPHandlerTask listens for incoming TCP data, places it in tcpToCanQueue.

  - CANHandlerTask receives this data, converts it to a CAN frame, and sends it on the CAN bus.

### WiFi and TCP Connection

- The program waits for a WiFi connection and then for packets from Rocrail.

- If the TCP connection to Rocrail is lost, the system will restart after 5 seconds.

### Debugging

- The debugFrame function provides detailed logging of CAN frames, including the frame's ID, length, and data.

- If VERBOSE is defined, it logs frames sent over the CAN network and TCP.

### Restart Logic

- If the CAN frame fails to send, the program attempts to send the frame up to 5 times.

- If the CAN sending process continues to fail, the system restarts.

- If Rocrail disconnects, the system restarts.

### Performance Considerations

- Task Priority ensures that the CAN and TCP message operations are given more CPU time.

- Core Pinning effectively balances workload across the two cores of the ESP32.

- Queues allow for asynchronous communication, enabling tasks to run in parallel without blocking each other.

Wi-Fi Manager

The WiFiManager class manages Wi-Fi connectivity for an ESP32 device, allowing it to connect to a network using stored credentials or set up a new connection in Access Point (AP) mode. This class simplifies managing Wi-Fi connections, offering a fallback for initial configuration and automatic reconnection.

- Startup: It attempts to connect to a saved Wi-Fi network using stored SSID and password. If this fails, it starts AP mode for manual configuration.

- AP Mode: In AP mode, it serves a webpage where the user can input new Wi-Fi credentials.

- Credentials Storage: The entered credentials are saved in the device's memory and used for future connections.

- Event Handling: It handles Wi-Fi disconnections and triggers a callback function if configured.

- Client Connection: It waits for client connections and handles them once connected.

- OTA updates: Updates can be performed prior to the client connection from the Arduino IDE.

- Telnet Debugging: The TelnetStream library is used for debug output. A telnet client connected to port 23 will see the debug output.

### Final Thoughts

- This design efficiently handles both CAN and TCP communications in a real-time, concurrent manner.

- The system ensures high priority for critical tasks while leaving room for debugging if needed.

- The use of queues and tasks helps decouple the processes, improving flexibility and performance